Physics 211 - Week 2

Two-Dimensional Kinematics: 2001

A Hollywood producer has decided to film a remake of the movie 2001: A Space Odyssey. You have been hired as a consultant for the movie to make sure the science is correct. The producer wanted to have an Illinois physics student for the job since HAL was invented in Urbana (January 12, 1997!). Part of the movie takes place on a space station very far from any gravitating body. This station is a large wheel-like structure where people live and work on the rim. In order to create "artificial gravity", the space station rotates about its axis.

One space station design is a large wheel-like structure where people live and work on the rim. In order to create "artificial gravity", the space station rotates about its axis. It is desired that the gravity be equal to 0.85 times that of the Earth. Since centripetal acceleration depends on the distance to the axis of rotation, it is not possible for the “artificial gravity” to be the same at the head and the feet of a standing person (say ~1.8 m). In order to minimize any possible discomfort, suppose that the difference in the “artificial gravity can only be 1%. The special effects department wants to know the diameter and the rate of rotation of a space station that meets these specifications.

The artificial gravity will be provided by the centripetal acceleration; we need to find both the diameter and the rate of rotation for the station.  Since we have two unknown quantities, we should try to create two equations to describe the motion of the station.  We know that the values of the artificial gravity at the head and the feet of a person should be within 1% of each other, so it is a good idea to set up one equation for each position.  Using the given height of the standing person, you should be able to develop a set of equations comparing the value of the artificial gravity to the centripetal acceleration for two positions- the head and the feet.  Solving them, you should obtain a diameter of 360m and a rotation rate of 0.034 revolutions per second.

1.8m

r = ?

f = ?