Walking the Plank

A uniform horizontal beam 8 m long is attached by a frictionless pivot to a wall.  A cable making an angle of 37o, attached to the beam 5 m from the pivot point, supports the beam, which has a mass of 600 kg.  The breaking point of the cable is 8000 N.  A man of mass 95 kg walks out along the beam.  How far can he walk before the cable breaks?