Physics 211		Week 10

Statics:  Walking the Plank (Solution)

A uniform horizontal beam 8 m long is attached by a frictionless pivot to a wall.  A cable making an angle of 37o, attached to the beam 5 m from the pivot point, supports the beam, which has a mass of 600 kg.  The breaking point of the cable is 8000 N.  A man of mass 95 kg walks out along the beam.  How far can he walk before the cable breaks?

Before the cable breaks, all of the torques and forces are in equilibrium.  Draw a free body diagram and use it to write net force and net torque equations.  Measuring torques about the given pivot will eliminate the unknown force from the pivot.  You’ll be left with torques from the weight of the plank, weight of the man, and the tension.  Using the maximum value of the tension as the measured value of the tensional force, you can find the maximum distance that the man can walk and still have his torque balanced so that the cable does not break.  You will find that he can walk 0.57 m.