Bowling Ball

Two identical bowling balls are rolling on a horizontal floor without slipping.  The initial speed of both balls is V = 9.9 m/s.  Ball A encounters a frictionless ramp, reaching a maximum vertical height HA above the floor. Ball B on the other hand rolls up a regular ramp (i.e. without slipping), reaching a maximum vertical height HB above the floor.   Which ball goes higher, and by how much?