Dumbbell Collision

A dumbbell consists of two balls, one of mass M and the other of mass 2M, connected by a light rod of length L.  The dumbbell is mounted vertically on a pivot with the heavier ball at the bottom.  The pivot is located at the midpoint of the rod.  The system, which is initially at rest, is free to rotate about the pivot.  A wad of putty of mass M and initial velocity V collides with and sticks to the lower mass, as shown in the diagram.  In terms of the quantities given, what is the minimum value of V for which the dumbbell will make it all the way around?