Physics 211 - Week 3

Force and Simple Dynamics: Hanging Sculpture (Solutions)

You are part of a team to help design the atrium of a new building.  Your boss, the manager of the project, wants to suspend a 10-kg sculpture high over the room by hanging it from the ceiling using thin, clear fishing line (string) so that it will be difficult to see how the sculpture is held up.  The only place to fasten the fishing line is to a wooden beam which runs around the edge of the room at the ceiling.  The fishing line that she wants to use is known to be able to support a vertically hanging mass of 10 kg, so she suggests attaching two lines to the sculpture to be safe.  Each line would come from the opposite side of the ceiling to attach to the hanging sculpture.  Her initial design has one line making an angle of 20o with the ceiling and the other line making an angle of 40o with the ceiling.  She knows you are taking physics, so she asks you if her design can work.

______________________________________________________________________

We can begin to find the maximum safe tension by finding the tension in the string when it vertically suspends the 10kg hanging mass.  Then we need to compare the tension in each of the two strings in the proposed design to the maximum tension to see if the sculpture will be secure.  The tensions in the two strings can be broken down into x and y components.  For the sculpture to remain horizontally stationary, the x-components of the tension in the two strings must have the same magnitude.  The y-components of the tensions both point in the upward direction; these components need to have a combined value equal to the weight of the sculpture.  Using this information to set up a system of equations you should be able to solve them and obtain values of 86.77 N for the 20o wire and 106.4 N for the 40o wire.  Since the fishing line she wants to use is only shown to support a tension of 98.1 N (found by vertically suspending the 10kg mass) and the 40o wire exceeds that tension, this design will not work.

20o

40o

mg

T20

T40