Ix

B

R2

I2

R3

I3

R4

R4

R1

Iy

E1

E2

A

R1 = 10 

R3 = 30 

E1 = 10 V

R2 = 15 

R4 = 25 

E2 = 30 V

1)	What is the relationship between the magnitudes of the currents Ix and Iy?  [4]

a)  |Ix| > |Iy| 		b)  |Ix| = |Iy| 		c)  |Ix| < |Iy|

2)	What is the current I2 through resistor R2? (A positive value means current flows in the direction of the arrow on the diagram.)  [5]

3)	What is the voltage difference VA – VB between points A and B?  [5]

4)	What is the current Iy? (A positive value means current flows in the direction of the arrow on the diagram.)  [6]