In the RLC circuit shown at right, the resistance R and capacitive reactance XC are set to the given values.  The inductor is of variable magnitude (indicated by the diagonal arrow).  The AC generator puts out a peak EMF of Emax = 120 V.

1)	If the inductance is set to L = 300 H, the circuit is in resonance.  Given this information, calculate the capacitance C of the circuit and the frequency  of the generator.  [5]

2)	Leaving the frequency  and peak voltage Emax of the AC generator unchanged, the variable inductor is now changed to L = 200 H.  This puts the circuit off resonance.  What is the phase angle  of the generator EMF with respect to the current?  [4]

3)	Does the generator EMF lead or lag the current when L = 200 H?  Provide a brief but clear reason for your answer.  [3]

4)	What is the impedance Z of the final circuit (with L = 200 H)?  [4]

5)	Draw the voltage phasor diagram (E, VC, VL, VR) for this final circuit, with the angles and lengths of the phasors indicated numerically.  [4]