E1

I2

E2

R2

R4

R1

R3

I1

B

C

A

R1 = 60 

R3 = 10 

E1 = 15 V

R2 = 6 

R4 = 20 

E2 = 10 V

1)	What is the relationship between the magnitudes of the currents I3 (through resistor R3) and I4 (through resistor R4)?  [4]

a)  |I3| > |I4|		b)  |I3| = |I4|		c)  |I3| < |I4|

Rubric:

Correct answer (4)

2)	What is the current I1 through resistor R1? (A positive value means current flows in the direction of the arrow on the diagram.)  [4]

E1 + E2 –I1R1 = 0

I1 = 0.42A

Rubric:

Correct problem setup (2)

Correct answer (2)

3)	What is the current I2 through resistor R2? (A positive value means current flows in the direction of the arrow on the diagram.)  [6]

E1 + E2 –I2 (R 1 + R34) = 0 where R34 is the equivalent resistance for R3 and R4, R34 = 6.7 ohms

I1 = 1.97A

Rubric:

Correct problem setup (4)

Correct answer (2)

4)	What is the potential difference VB – VC between points B and C?  [6]

VB – VC = V2 – I2R2 = -1.84 V

Rubric:

Correct problem setup (4)

Correct answer (2)