x x x x

x x x x

x x x x

x x x x

v'

v

1)	A proton (mass m, charge q) moving at velocity v enters a region of constant magnetic field B directed into the paper as shown.  The proton exits the magnetic field region with velocity v’ as shown.  Compare vx’ (the x-component of this final velocity) with the initial velocity v.  [4]

(a)  vx’  <  v		(b)  vx’  =  v		(c)  vx’  >  v

In the circuit below, the switch is initially open (as shown) and the capacitors are uncharged.

R1

R2

C1

C2

E

R1 = 4  C1 = 40 F

R2 = 2      C2 = 20 F

E = 20V

2)	What is the current I1 through resistor R1 immediately after the switch is closed?  [3]

3)	What is the current I1 through resistor R1 after the switch has been closed for a long time?  [3]

R1

R2

C1

C2

E

R1 = 4  C1 = 40 F

R2 = 2      C2 = 20 F

E = 20 V

4)	What is the charge Q1 on capacitor C1  a long time after the switch is closed? You will receive full credit for a correct algebraic or numerical answer.  [6]

5) After a long time, the capacitors are fully charged and the switch is reopened.  What is the time constant  for discharging the capacitors?  Specify your answer in units of seconds.  [4]