Shown below are mathematical and pictorial representations of an electromagnetic plane wave propagating through empty space.  The electric field is parallel to the z-axis.

E(x, y, z, t) = Eo sin(kx + t)z

The points R, and S in the diagram above lie in the x–y plane, the points P and Q also lie in the x–y plane.  Each of the questions relates to the instant shown.

- Which statement below correctly relates the magnitude of the magnetic field at points P and Q? [3]

a) BP > BQ  > 0		b) BP > BQ = 0		c) BP = BQ > 0

d) BP = BQ = 0 		e) BQ > BP > 0		f) BQ > BP = 0

- Explain your reasoning. [2]

- Which statement below correctly relates the magnitude of the magnetic field at points S and R? [3]

a) BS > BR > 0		b) BS > BR = 0		c) BS = BR > 0

d) BS = BR = 0		e) BR > BS > 0 	f) BR > BS = 0

- Explain your reasoning. [2]

- Write an expression for the magnetic field B(x,y,z,t).  (Remember to express the amplitude of the magnetic field in terms of the amplitude of the electric field.) [4]

- In which direction is the wave traveling? [3]

- Suppose that the diagram above represents a radio wave.  In order to obtain the best reception with a radio, how would you orient the antenna?  Remember, the optimal orientation to maximize reception is with the antenna parallel (or antiparallel) to the electric field. [3]

a) parallel to the x-axis

b) parallel to the y-axis

c) parallel to the z-axis

d) parallel to the y-z plane and at an angle of 45° with respect to the y-axis

e) any orientation parallel to the y-z plane.