Physics 214	Problem 5	Week 2

# Phased Array

d

d

d

We can use interference effects to make a very directional antenna (either for transmitting or receiving).  As an example, consider the following situation: 4 radio antennas are equally spaced in a line, separated by intervals of length d.  The antennas all emit radiation coherently, with the same intensity I1 and the same wavelength .  Initially the antennas all emit in phase.

1.  For concreteness, assume d = /4.  Draw phasor diagrams corresponding to the net amplitude far away from the antennas in the directions indicated.   Calculate the intensity in each of these directions.  Hint: In considering radiation to the east, the signal from antenna #1 takes longer to arrive, so its phasor will lag behind that of antenna #2, which will lag behind that of antenna #3, etc.

2.  Since d < , there are no other principal maxima except along the N and S directions.  Why?

Thus, with four antennas, the power is emitted primarily into two lobes (even though each antenna individually is emitting into all directions).  Similarly, if this array were acting as a receiver, it would primarily detect signals from the North and South.  By using more antennas, the directionality would become sharper.

Now we use slightly more sophisticated electronics to introduce a relative phase /2 between adjacent antennas, as shown below.  That is, if the time-dependence of antenna 1 is cos(t), the time-dependence of antenna 2 is cos(t + /2), the time-dependence of antenna 3 is cos(t + ), etc.  We will see that this has the effect of changing the direction of the primary radiation lobes.

3.  Draw new phasor diagrams corresponding to the net amplitude far away from the antennas in the directions indicated.   Calculate the intensity in each of these directions.