Physics 214	Problem 2	Week 1

Solution

- Traveling Wave Pulse

A voltage pulse travels down a coaxial cable in the +x direction at a speed of v = 2.5  108 m/s.

At time t = 0 the potential difference between the inner conducting wire and the outer conducting shell is given by this formula,V(x)=V0e−αx2, where Vo = 0.7 volts and = 2 m-2.  (Note: x2 is dimensionless.)

x

Vo

V (Volts)

x (m)

-2

2

1

0

-1

On the axes below, sketch V(x) versus x showing the shape of the pulse at time t = 0 s.  (Hint: For what values of x does the curve fall to 1/e of its maximum value?  xe =  ±0.707m) With a dashed curve, sketch the pulse at a time 4 nanoseconds later. At t = 4 ns, the maximum potential occurs at what value of x?   xmax = vt = 1 m.

- V(x)=Voe−α(x−1m)2Give a formula for the new function V(x) that describes the pulse at t = 4 ns.

- V(x,t)=Voe−α(x−vt)2Write a formula for V(x,t) that describes the pulse as a function of position and time. (You may use v for velocity.)

- V(x)=Voe−α(x+vt)2What formula would describe the pulse if it were going in the opposite direction?

- e)   Finally, consider two Gaussian pulses traveling down the cable in opposite directions, and with equal magnitudes, but opposite signs (i.e., the dashed lines shown represent the separate pulses). Draw the net voltage along the cable for the three times shown:

- Good students may wonder where the energy resides at the time when the displacements cancel. It’s in the velocities, which interfere constructively.

t = 0

t < 0