Physics 214	Problem 2	Week 4

Solution

Electron Microscope

From optics we know that, no matter how good the instrument is, the resolution will be limited by diffraction.  The Rayleigh criterion, sin D, says that one cannot resolve an object smaller than approximately x = 1.22 f/D, where f is the focal length of the lens, D is the diameter of the lens, and  is the wavelength of the illuminating radiation.*  Electron microscopes have become important because the de Broglie wavelength of an electron can be much smaller than the wavelength of  visible light, making it possible to resolve objects with nanometer resolution.  In practice, the ratio f/D in an electron microscope is about 100.

- What wavelength electrons are needed in an electron microscope, if we wish to resolve two atoms in a crystal separated by x = 0.3 nm?

 = (x  D/f ) / 1.22 = 0.00246 nm

- What voltage is needed to accelerate electrons enough to get that wavelength, assuming that the non-relativistic relation, E = p2/2m. between momentum and energy holds?

E = h2/2m2 = 1.505eV-nm2/2  Voltage = 249 kV

*  For camera enthusiasts, f/D is called the “f-number” (written “f/#”) of a lens.  Good cameras can have f/#’s down to f/4.  About the best you can do is f/1.