Physics 214	Problem 5	Week 2

Solution

# Phased Array

d

d

d

We can use interference effects to make a very directional antenna (either for transmitting or receiving).  As an example, consider the following situation: 4 radio antennas are equally spaced in a line, separated by intervals of length d.  The antennas all emit radiation coherently, with the same intensity I1 and the same wavelength .  Initially the antennas all emit in phase.

1.  For concreteness, assume d = /4.  Draw phasor diagrams corresponding to the net amplitude far away from the antennas in the directions indicated.   Calculate the intensity in each of these directions. Hint: In considering radiation to the east, the signal from antenna #1 takes longer to arrive, so its phasor will lag behind that of antenna #2, which will lag behind that of antenna #3, etc.

N,S:  If we are far away, then in the up (N) or down (S) directions, all the path lengths are essentially equal.  Therefore, we have 

Atot = 4A1   Itot = 16I1

E:  In this case,  r1 – r2 = dTherefore, 

Atot = 0  Itot = 0.

A2

A3

A4

A1

W:  In this case,   r1 – r2 =dTherefore, 

A1

A2

A3

A4

Atot = 0  Itot = 0.

To the west, the antenna #2 signal lags the antenna #1 signal, so A2 will lag behind A1, and so on.

2.  Since d < , there are no other principal maxima except along the N and S directions.  Why?

The only way to get a principal maximum is if the path length difference is an integer multiple of the wavelength.  Since d is less than one wavelength, there is no way to make  = ..

Thus, with four antennas, the power is emitted primarily into two lobes (even though each antenna individually is emitting into all directions).  Similarly, if this array were acting as a receiver, it would primarily detect signals from the North and South.  By using more antennas, the directionality would become sharper.

Now we use slightly more sophisticated electronics to introduce a relative phase /2 between adjacent antennas, as shown below.  That is, if the time-dependence of antenna 1 is cos(t), the time-dependence of antenna 2 is cos(t + /2), the time-dependence of antenna 3 is cos(t + ), etc.  We will see that this has the effect of changing the direction of the primary radiation lobes.

3.  Draw new phasor diagrams corresponding to the net amplitude far away from the antennas in the directions indicated.   Calculate the intensity in each of these directions.

N,S:  As before, 0.  But now each phasor is rotated counterclockwise by i

A2

Atot = 0  Itot = 0.



A1

A3

A4

E:  We start with the East-solution from part 1, i.e., due to the distance effect, each phasor is rotated /2 counterclockwise with respect to the previous one:

A2

A3

A4

A1

However, due to the phase offset  = (n-1), each phasor is rotated counterclockwise an additional (n-1)/2.  The net effect is that each phasor has a relative phase of  with respect to the previous one:

Atot = 0  Itot = 0.

A2

A3

A4

A1

W: Again we start with the West-solution from part 1, i.e., due to the distance effect, each phasor is rotated /2 clockwise with respect to the previous one:

A1

A2

A3

A4

The phase offset still rotates each phasor counterclockwise (n-1)/2.  Now all the phasors line up:

Atot = 4A1   Itot = 16I1

A1

A2

A3

A4

Here’s a mathematical calculation of the waves everywhere; green denotes the nodes, red denotes the peaks and blue denotes the troughs. The cancellation over the entire eastern hemisphere of the plot is quite dramatic. If instead the phases of the antennas were , the power would be emitted to the east.  By using other values of , the direction of the emitted power can be varied.  This effect is used in many radio telescope arrays for astronomy.