Transportation Infrastructure Precast Innovation Center

(TRANS-IPIC)

University Transportation Center (UTC)

Optimizing Precast Concrete Supply Logistics Systems Under Variable Demand And Probabilistic Delivery Time To Enhance Reliability, Sustainability And Cost-Effectiveness – Phase II (2024)

[Y1: LS-23-RP-04/Y2: LS-24-RP-01]

LSU Proposal ID: AWD-005947, GR-00016909

Quarterly Progress Report: QPR Y2-3

Performance period: July 1- September 30, 2025

Submitted by:

Bhaba R. Sarker (PI)

Professor, Department of Mechanical and Industrial Engineering

Louisiana State University, Baton Rouge, LA 70803

bsarker@lsu.edu

Collaborators / Partners:

Advisor: 	Dr. Tyson Rupnow, Associate Director, Louisiana Transportation Research Center (LTRC).

Participating Co.: 	Rinker, Gainey’s, Premier Concrete Products (PCP), and WASKEY for collaboration and validation of research works.

Graduate Assistant:  Anik Mazumder

Submitted to:

TRANS-IPIC UTC

University of Illinois Urbana-Champaign

Urbana, IL

TRANS-IPIC Quarterly Progress Report (Y2-3)

- PROJECT DESCRIPTION

The role of precast concrete (PC) supply logistics under variable demand and probabilistic delivery times is explored here to enhance reliability, sustainability, and cost-effectiveness. In this section, we highlight the problem statement and the project’s tasks to be completed.

- Research Plan - Statement of Problem

Reliability and sustainability of transportation infrastructure depend significantly on the optimal scheduling and routing of precast concrete (PC) resources, particularly in delivery operations (as experienced by companies like Gainey’s, WASKEY, and Premier Concrete Products in Louisiana). Efficient transportation ensures timely delivery of PC products, reducing project delays and preventing costly material degradation. Thus, this USDOT Trans-IPIC project focuses on developing cost-effective strategies to enhance reliability and sustainability in PC distribution systems under variable demand and probabilistic delivery times. The primary goal of this research is to provide a scalable solution that not only reduces the logistic costs associated with PC supply but also enhances the reliability and sustainability in facing variable demands and delivery times in constructions of transportation infrastructures and its logistical challenges.

- Research Plan - Summary of Project Activities (Tasks)

As outlined in the proposed research plan for Year 1, the model addressing uncertainty and price volatility (UPV) in the PC supply network is in progress. In addition to that, probabilistic travel time and stochastic demand, which are quite common phenomena in transportation construction, are to be considered for achieving the current objective of Year 2 (2025) research undertaken and these include:

- Task 1: ANM (Activity Network Model): It will highlight a proper representation of predecessors and successors related to PC production activities incorporating probabilistic travel time and stochastic demand with price fluctuation, to determine production and delivery times.

- Task 2: IOTC (In-plant and Off-plant Transportation Cost): The IOTC model will optimize storage location/assignment to enhance utilization and maximize transportation costs for precast components from plants to construction sites.

- Task 3: DSC (Delivery System Cost): The model will bring forth a shed on the general behavior of PC delivery schedules, minimize penalty costs, address material uncertainty and price volatility, ensuring reliable and sustainable PC supply systems.

- PROJECT PROGRESS

The progress in completing the current research tasks is highlighted here, followed by future work, educational outreach, workforce development and technology transfer.

- Progress for each research task

The total task of Year 2 were partitioned into three major segments: ANM, IOTC and DSC with an intended quarterly schedule for each of them, followed by a final report for the last quarter.

3.1 Task 1 (QPR Y2-1): Activity Network Modeling

Efficient logistics and delivery systems for PC components are critical for successful road and highway construction projects, significantly influencing overall project reliability, sustainability, and cost-efficiency. A proper process flow mapping approach is required to represent a systematic framework for analyzing logistical processes, spanning from procurement planning to the final delivery at construction sites.

Figure 1. Activity flow for PC shipments to construction sites

Figure 1 highlights three stages, procurement, yard layout, and delivery—where optimization ensures consistent material quality, proper curing, and timely placement. Consideration of these 3 signifucant factors can reduce delays and enhances the durability and long-term performance of precast roads and highways. For example, the cooperative procuring policy developed by Mazumder and Sarker (2025b) proved that by optimizing the shipment schedules of perishable precast raw materials under variable demand and travel time scenarios, both the vendor and precast manufacturer can get benefitted and delays in PC components delivery can be reduced.

Table 1. Optimal solutions for uniform travel time and exponential demand situations

| Number of Shipments, n | Order Quantity, Qn (tons of PPC) | Reorder point, rn (tons of PPC) | TC ($/year) |
| --- | --- | --- | --- |
| 1 | 554 | 283 | 60,849.74 |
| 2 | 364 | 284 | 60,186.94* |
| 3 | 285 | 284 | 62,124.08 |

Table 1 shows that the PC manufacturer faces minimum integrated inventory cost of $60,186.94 when 2 shipments are sent from a vendor to the manufacturer, each carrying 364 tons of Portland Pozzolana Cement and the manufacturer will place a new order when their current inventory reduces to 284 units.

(a) Efficient PC material handling

(b) Graphical sensitivity analysis

Figure 2. Optimal procuring in a typical PC manufacturing facility

Figure 2b shows how changing four key parameters Am (manufacturer setup cost), Av (vendor ordering cost), p0 (unit revenue loss), and cp (perishability penalty) affects total integrated cost. Am and Av have the steepest curves, meaning they significantly impact total cost, especially Av. p0 has a moderate effect, while cp shows minimal influence. Overall, setup costs drive most of the cost variation, while penalty-related parameters have a smaller impact.

3.2 Task 2 (QPR Y2-2): IOTC (In-plant and Off-plant Transportation Cost)

(a)

(b)

Figure 3. One way truck path at PCP

Efficient yard layout and crane operations are vital to prevent damage to precast components during curing and handling. At Premier Concrete (PCP), time-motion studies and truck flow analysis led to one-way routing, storage zones, and FIFO inventory (Figure 3a), reducing congestion and improving access. At Waskey, GPS crane data and BlueBeam simulations guided linear pathways, sector assignments, and staging zones (Figure 3b), cutting crane idle time and handling risks. These optimizations improved efficiency, durability, and reliability of precast operations. By addressing site-specific issues at PCP and Waskey, we achieved measurable improvements: truck path deviation time at PCP was reduced by 18%, while Waskey saw a 15% decrease in crane travel time, 10% less stoppage, and up to 30% better material handling efficiency. These changes enhanced workflow, reduced risks, and protected the structural integrity of precast components.

3.3 Task 3 (QPR Y2-3): DSC (Delivery System Cost)

Finished precast concrete products must be delivered to various construction sites with differing quantity, type, and timing requirements. This overall process (Figure 4) involves selecting the right inventory and optimizing delivery routes. The overall supply chain from raw material procurement to final delivery faces logistical challenges such as minimizing transportation costs, meeting deadlines, and maintaining product quality.

Figure 4. Shipment activities from precast plants to construction sites

The research objective illustrated Figure 5 is to develop and optimize a Precast Concrete Delivery (PCD) model that supports strategic and operational decision-making under uncertainty.

Figure 5. Model PCD

This PCD model was run by some collected data (Ref. [45]-[53]) to check its validity, and Figure 6 summarizes key parameters such as project deadlines, and the range of batch demands that the model must satisfy.

,

Figure 6. PC components requirements in multiple projects

Figure 7. Average usage of two transporters

This large dataset was applied in our PCD model for optimizing allocation different transportation modes under multiple uncertain demand and travel time scenarios.

Total 10 projects data from multiple highways and bridges construction were considered for initial analysis where different demand and travel time scenarios were assumed to represent uncertainty. Again, construction project of a certain bridge does not require PC components all the time since there are requirements of several in-site concrete products for such construction projects. Therefore, there are several scenarios where for any specific construction project, there is no demand for any PC components in any lot. Figure 8 shows precast components moving from the PC manufacturer to the construction site using two transport modes. A flatbed makes 8 trips at about 88% of its capacity, while a standard truck makes 5 trips at about 97% (nearly full loads). Together, these multi-modal shipments carry the required pieces from the plant to the site efficiently.

Figure 8. Optimal allocation of the two transportation modes

Figure 9 shows how the genetic algorithm’s (GA) best cost improves over generations (computational efficiency). It starts high (about 1.32×10⁵) and drops quickly in the first 8–12 generations, meaning the algorithm finds much better solutions early on. After that, the curve flattens near 1.15×10⁵ around generation 15, with only tiny gains afterward and a small dip again near generation 39. In plain terms: big improvements happen early, then progress slows a lot, so the search has essentially converged by about 15–20 generations.

Figure 9. Cost convergence to minimized value over generations

- Percent of research project completed.

As the research project for this year has been divided into three parts, the percentage of the research project completed is approximately (95%×33.33%) + (85%×33.33%) + (85%×33.33%) ~ 88%. The computational parts of Stage 2 and 3 are in progress, and they will be included in the working paper to be submitted to a refereed journal.

- Expected progress for next quarter.

Over the next quarter, we plan to integrate more types of transportations into the model and run pilot tests to assess its performance under practical conditions. We will refine the solution approach to handle new constraints that arise from these tests, aiming to enhance the model’s robustness and efficiency.

- Educational outreach and workforce development

A total of 12 undergraduate (UG) students has been assigned to the following research projects in their respective industries as part of their capstone course requirement for IE-4598: Senior Design Project II (Spring 2025). Tracking and storing Besser Machines parts for precast concrete manufacturing (project PCP-1).

- Optimizing the yard layout to improve truck flow in transporting precast materials (project PCP-3).

- Improving yard layout for efficient handling and tracking of precast concrete products (Waskey).

- Design of dunnage and rigging methods for oversized precast concrete: ensuring stability and minimizing damage (Gainey’s).

- Technology Transfer

The research team is in contact with several companies [Precast Concrete Products (PCP), Waskey, Gainey’s and Rinker] through LTRC (Louisiana Transportation Research Center). Efforts have been made to collaborate and collect some real-life data for testing purposes and feedback on the project. Some of this data is expected to be incorporated in subsequent reports. The outcome is yet to be achieved for technology transfer.

- RESEARCH CONTRIBUTION

- Papers that include TRANS-IPIC UTC in the acknowledgments section:

      - Mazumder, A. and Sarker, B.R. (2025a), “Determining optimal variable order quantities of raw materials for precast concrete production considering demand variability and uncertain material prices," Paper-1 (LS-23-RP-04), Submitted to Asia Pacific Journal of Operational Research. 1st submission: September 8, 2025 (APJOR-S-25-00647).

      - Mazumder, A. and Sarker, B. R. (2025b), “Optimal vendor-manufacturer cooperative policy for perishable precast materials under stochastic demand and lead-time uncertainty,” Paper-4 (LS-23-RP-04), Submitted to International Journal of Production Economics (under review), August 19, 2025 (PROECO-D-25-02726)

      - Mazumder, A. and Sarker, B. R. (2025c), “Optimal Vehicle and Resource Allocation for Precast Concrete Transportation under Demand and Travel Time Uncertainty,” Working Paper-3 (LS-24-RP-01), Intended for ASCE: Journal of Transportation Engineering.

- Presentations and Posters of TRANS-IPIC funded research:

      - Mazumder, A. and Sarker, B. R. (2024), “Factors of the Precast Concrete Supply Chain: An Interpretive Structural Modeling Approach,” Graduate Research Conference (GRC), presented at the Students’ Union, Louisiana State University, Baton Rouge, LA on April 30, 2024.

      - Mazumder, A. and Sarker, B. R. (2025a), “Optimizing Raw Material Ordering Policies for Efficient Precast Concrete Production under Fluctuating Demand and Price,” Presentation ID: 6670, IISE Annual Conference & Expo 2025, Renaissance Atlanta Waverly Hotel & Convention Center (Room: Tyndall), Atlanta, Georgia, May 31- June 3, 2025.

      - Mazumder, A. and Sarker, B. R. (2025b), “Optimal Component Allocation on Pallet to Minimize Curing Costs in Precast Manufacturing,” Presentation ID: 8864, IISE Annual Conference & Expo 2025, Renaissance Atlanta Waverly Hotel & Convention Center (Room: Andover), Atlanta, Georgia, May 31- June 3, 2025.

      - Mazumder, A. and Sarker, B. R. (2025c), “Optimizing Pallet Utilization to Reduce Curing Costs in Precast Concrete Manufacturing,” Poster ID: 7037, 2025 INFORMS Annual Meeting, Georgia World Congress Center and Omni Atlanta Hotel at Centennial Park, Atlanta, Georgia, October 26- 29, 2025.

Interim Report presentations at LSU (PFT-1206) on March 21, 2025, by the undergraduate students who are working in the precast manufacturing industries:

- PCP-1: Tracking and storing Besser Machines parts for precast concrete manufacturing (3 students).

- PCP-3: Optimizing the yard layout to improve truck flow in transporting precast materials (3 students).

- Waskey: Improving yard layout for efficient handling and tracking of precast concrete products (3 students).

- Gainey’s: Design of dunnage and rigging methods for oversized precast concrete: ensuring stability and minimizing damage (3 students).

- Please list any other events or activities that highlights the work of TRANS-IPIC occurring at your university (please include any pictures or figures you may have). Similarly, please list any references to TRANS-IPIC in the news or interviews from your research.

      - Images from interim report presentation slides on March 21, 2025, by the undergraduate students who are working in the precast manufacturing industries.

      - Mazumder, A. and Sarker, B. R., “Factors of the Precast Concrete Supply Chain: An Interpretive Structural Modeling Approach,” Graduate Research Conference (GRC), presented at the Students’ Union, Louisiana State University, Baton Rouge, LA on April 30, 2024.

Mazumder, A. and Sarker, B. R., “Optimizing Pallet Capacity Utilization to Minimize Curing Cost in Precast Concrete Manufacturing,” presented at the DOT TRANS-IPIC online research online seminar on July 22, 2024.

      - Plant tour and meeting to discuss potential challenges in precast concrete logistics for the TRANS-IPIC project proposal for Year 2, held at Rinker Materials in Alexandria, LA, on September 23, 2024.

      - Presentations cover slides at IISE Annual Conference & Expo 2025.

Appendix 1

Research Activities, leadership, and awards (cumulative, since the start of the project)

- Number of presentations at academic and industry conferences and workshops of UTC findings

- No. = 4

- Number of peer-reviewed publications submitted based on outcomes of UTC funded projects.

- No. = 2

- Number of peer-reviewed journal articles published by faculty.

- No. = 0

- Number of peer-reviewed conference papers published by faculty.

- No. = 0

- Number of TRANS-IPIC sponsored thesis or dissertations at the MS and PhD levels.

- No. MS thesis = 1 (Thesis defense scheduled on October 29, 2025)

- No. PhD dissertations = 1 (on-going)

- No. citations of each of the above = 0

- Number of research tools (lab equipment, models, software, test processes, etc.) developed as part of TRANS-IPIC sponsored research.

- Research Tool #1 (Name, description, and link to tool) = 0

- Research Tool #2 (Name, description, and link to tool) = 0

- Research Tool #3 (Name, description, and link to tool) = 0

- Number of transportation-related professional and service organization committees that TRANS-IPIC faculty researchers participate in or lead.

- Professional societies

  - No. participated in = 4 (DSI, IISE, INFORMS, POMS)

  - No. lead = 0

- Advisory committees (No. participated in and No. led)

  - No. participated in = 1 (IISE Fellow)

  - No. lead = 0

- Conference Organizing Committees (No. participated in and No. led)

  - No. participated in = 0

  - No. lead = 0

- Editorial board of journals (No. participated in and No. led)

  - No. participated in = 2

  - No. lead = 0

- TRB committees (No. participated in and No. led)

  - No. participated in = 0

  - No. lead = 0

- Number of relevant awards received during the grant year.

- No. awards received = 0

- Number of transportation related classes developed or modified as a result of TRANS-IPIC funding.

- No. Undergraduate = 0

- No. Graduate = 0

- Number of internships and full-time positions secured in the industry and government during the grant year.

- No. of internships = 0

- No. of full-time positions = 0

REFERENCES

- Allison, J. (1990). Combining Petrov's heuristic and the CDS heuristic in group scheduling problems. Computers and Industrial Engineering, 19(1), 212–215.

- Anvari, B., Angeloudis, P., and Ochieng, W. (2016). A multi-objective GA-based optimisation for holistic manufacturing, transportation, and assembly of precast construction. Automation in Construction, 71, 226–241.

- Benjaoran, V., Dawood, N., and Hobbs, B. (2005). Flowshop scheduling model for bespoke precast concrete production planning. Construction Management and Economics, 23(2), 93–105.

- Chan, W., and Hu, H. (2000). Precast production scheduling with genetic algorithms. Proceedings of the 2000 Congress on Evolutionary Computation, 2, 1087–1094.

- Chan, W., and Hu, H. (2001). GA-based resource-constrained flow-shop scheduling model for mixed precast production. Automation in Construction, 11(4), 439–452.

- Chan, W., and Hu, H. (2002). Production scheduling for precast plants using a flow shop sequencing model. Journal of Computing in Civil Engineering, 16(3), 165–174.

- Chen, J.-H., Hsu, S., Chen, C.-L., Tai, H.-W., and Wu, T.-H. (2020). Exploring the association rules of work activities for producing precast components. Automation in Construction, 111, 103059.

- Chen, J.-H., Yan, S., Tai, H.-W., and Chang, C. (2017). Optimizing profit and logistics for precast concrete production. Canadian Journal of Civil Engineering, 44(5), 393–406.

- Chen, J.-H., Yang, L.-R., and Tai, H.-W. (2016). Process reengineering and improvement for building precast production. Automation in Construction, 68, 249–258.

- Dan, Y., Liu, G., and Fu, Y. (2021). Optimized flowshop scheduling for precast production considering process connection and blocking. Automation in Construction, 125, 103575.

- Hu, H. (2007). A study of resource planning for precast production. Architectural Science Review, 50(2), 106–114.

- Jiang, W., and Wu, L. (2021). Flow shop optimization of hybrid make-to-order and make-to-stock in precast concrete component production. Journal of Cleaner Production, 297, 126708.

- Khalili, A., and Chua, D. (2014). Integrated prefabrication configuration and component grouping for resource optimization of precast production. Journal of Construction Engineering and Management, 140(8), 04013052.

- Kim, T., Kim, Y.-W., and Cho, H. (2020). Dynamic production scheduling model under due date uncertainty in precast concrete construction. Journal of Cleaner Production, 257, 120527.

- Ko, C., and Wang, S.-F. (2010). GA-based decision support systems for precast production planning. Automation in Construction, 19(7), 907–916.

- Ko, C., and Wang, S.-F. (2011). Precast production scheduling using multi-objective genetic algorithms. Expert Systems with Applications, 38(7), 8293–8302.

- Kong, L., Li, H., Luo, H., Ding, L., and Skitmore, M. (2017). Optimal single-machine batch scheduling for the manufacture, transportation, and JIT assembly of precast construction with changeover costs within due dates. Automation in Construction, 81, 34–43.

- Kong, L., Li, H., Luo, H., Ding, L., and Zhang, X. (2018). Sustainable performance of just-in-time (JIT) management in time-dependent batch delivery scheduling of precast construction. Journal of Cleaner Production, 193, 684–701.

- Leu, S.-S., and Hwang, S.-T. (2001). A GA-based model for maximizing precast plant production under resource constraints. Engineering Optimization, 33(6), 619–642.

- Leu, S.-S., and Hwang, S.-T. (2002). GA-based resource-constrained flow-shop scheduling model for mixed precast production. Automation in Construction, 11(4), 439–452.

- Li, S. H. A., Tserng, H., Yin, S. Y. L., and Hsu, C.-W. (2010). A production modeling with genetic algorithms for a stationary pre-cast supply chain. Expert Systems with Applications, 37(12), 8406–8416.

- Li, Z., Shen, G., and Xue, X. (2014). Critical review of the research on the management of prefabricated construction. Habitat International, 43, 240–249.

- Liao, T. W, Egbelu, P. J., Sarker, B. R. and Leu, S. S. “Metaheuristics for project and construction management—A state-of-the-art review,” Automation in Construction, 20 (5): August 2011, pp. 491-505.

- Liu, Y., Dong, J., and Shen, L. (2020). A conceptual development framework for prefabricated construction supply chain management: An integrated overview. Sustainability, 12(5), 1878.

- Liu, Z., Liu, Z., Liu, M., and Wang, J. (2021). Optimization of Flow Shop Scheduling in Precast Concrete Component Production via Mixed‐Integer Linear Programming. Advances in Civil Engineering, 2021(1), 6637248.

- Ma, Z., Yang, Z., Liu, S., and Wu, S. (2018). Optimized rescheduling of multiple production lines for flowshop production of reinforced precast concrete components. Automation in Construction, 95, 86–97.

- Mazumder, A. and Sarker, B.R. (2025a), “Determining optimal variable order quantities of raw materials for precast concrete production considering demand variability and uncertain material prices," Paper-1 (LS-23-RP-04), Submitted to Asia Pacific Journal of Operational Research. 1st submission: September 8, 2025 (APJOR-S-25-00647).

- Mazumder, A. and Sarker, B. R. (2025b), “Optimal vendor-manufacturer cooperative policy for perishable precast materials under stochastic demand and lead-time uncertainty,” Paper-4 (Gen Demand), Submitted to International Journal of Production Economics (under review), August 19, 2025 (PROECO-D-25-02726)

- Nori, V. S., and Sarker, B. R. (1996). Cyclic scheduling for a multi-product, single-facility production system operating under a just-in-time delivery policy. Journal of the Operational Research Society, 47(7), 930-935.

- Parija, G. R., and Sarker, B. R. (1999). Operations planned in a supply chain system with fixed-interval deliveries of finished goods to multiple customers. IIE transactions, 31(11), 1075-1082.

- Prata, B., Pitombeira-Neto, A. R., and Sales, C. J. de M. (2015). An integer linear programming model for the multiperiod production planning of precast concrete beams. Journal of Construction Engineering and Management, 141(9), 04015029.

- Ruan, M., and Xu, F. (2022). Improved eight-process model of precast component production scheduling considering resource constraints. Journal of Civil Engineering and Management, 28(3), 1–15.

- Sarker, B. R., and Parija, G. R. (1994). An optimal batch size for a production system operating under a fixed-quantity, periodic delivery policy. Journal of the Operational Research Society, 45(8), 891-900.

- Sarker, B. R., and Parija, G. R. (1996). Optimal batch size and raw material ordering policy for a production system with a fixed-interval, lumpy demand delivery system. European Journal of Operational Research, 89(3), 593-608.

- Sarker, B. R., Egbelu, P. J., Liao, T. W. and Yu, J., “Planning and design models for construction Industry: A critical survey,” Automation in Construction, 22(SI-1), March 2012, pp. 123-134.

- Wang, D., Liu, G., Li, K., Wang, T., Shrestha, A., Martek, I., and Tao, X. (2018). Layout optimization model for the production planning of precast concrete building components. Sustainability, 10(6), 1807.

- Wang, Z., and Hu, H. (2017). Improved precast production-scheduling model considering the whole supply chain. Journal of Computing in Civil Engineering, 31(6).

- Wang, Z., Hu, H., and Gong, J. (2018). Framework for modeling operational uncertainty to optimize offsite production scheduling of precast components. Automation in Construction, 86, 69–80.

- Wang, Z., Hu, H., and Gong, J. (2018). Modeling worker competence to advance precast production scheduling optimization. Journal of Construction Engineering and Management, 144(12), 04018109.

- Wang, Z., Hu, H., and Gong, J. (2018). Simulation based on multiple disturbances evaluation in the precast supply chain for improved disturbance prevention. Journal of Cleaner Production, 177, 232–244.

- Wang, Z., Hu, H., and Gong, J. (2019). Precast supply chain management in off-site construction: A critical literature review. Journal of Cleaner Production, 232, 1104–1116.

- Wang, Z., Hu, H., Gong, J., and Ma, X. (2018). Synchronizing production scheduling with resources allocation for precast components in a multi-agent system environment. Journal of Manufacturing Systems, 49, 109–120.

- Wang, Z., Liu, Y., Hu, H., and Dai, L. (2021). Hybrid rescheduling optimization model under disruptions in precast production considering real-world environment. Journal of Construction Engineering and Management, 147(5), 04021012.

- Yang, Z., Ma, Z., and Wu, S. (2016). Optimized flow shop scheduling of multiple production lines for precast production. Automation in Construction, 72, 321–329.

- Yin, Y., Cheng, S.-R., and Wu, C.-C. (2014). Parallel-machine scheduling to minimize flowtime, holding, and batch delivery costs. Asia-Pacific Journal of Operational Research, 31(6), 1450044.

- Zhang, H., and Yu, L. (2020). Dynamic transportation planning for prefabricated component supply chain. Engineering, Construction and Architectural Management, 27(10), 2553–2576.

- ATS Inc. (2025). How much weight can I put on a flatbed trailer? https://www.atsinc.com/blog/how-much-weight-can-flatbed-trailers-carry-kingpin-laws-explained

- Freedom Heavy Haul. (2025). Flatbed trailer weight limits. https://freedomheavyhaul.com/flatbed-trailer-weight-limits/

- Sonar. (2025). How is freight priced? A dive into freight rates by mode of transit. https://gosonar.com/freight-market-blog/how-is-freight-priced-a-dive-into-freight-cost-per-unit-of-measurement

- Method CRM. (2025). Freight rates: Trucking rates per mile 2024. https://www.method.me/pricing-guides/trucking-rates-per-mile/

- Bureau of Transportation Statistics. (2025). Average truck speeds on select metropolitan area interstates. https://www.bts.gov/browse-statistical-products-and-data/freight-facts-and-figures/average-truck-speeds-select

- Omni Calculator. (2025). Drive time calculator – travel time calculator. https://www.omnicalculator.com/everyday-life/drive-time

- Federal Highway Administration. (2025). Estimating handbook, https://highways.dot.gov/media/47116

- Alobees. (2025). Penalties for late construction site delivery explained. https://www.alobees.com/en/conseils/penalties-late-construction

- Stahlton Prestressed Concrete. (2025). Panels. https://stahlton.co.nz/products/panels/