Transportation Infrastructure Precast Innovation Center

(TRANS-IPIC)

University Transportation Center (UTC)

AI-based Lift Path Planning for Robotic Installation of Precast Bridge Components

UT-25-RP-01

Quarterly Progress Report

For the performance period ending 03/31/2026

Submitted by:

PI: Dr. Jiannan Cai, jiannan.cai@utsa.edu

School of Civil & Environmental Engineering, and Construction Management (CECM)

The University of Texas at San Antonio (UTSA)
Co-PI: Dr. Ao Du, ao.du@utsa.edu,CECM, UTSA

Graduate research assistant: Mohsen Navazani, Mohsen.Navazani@utsa.edu, CECM, UTSA

Collaborators / Partners:

N/A

Submitted to:

TRANS-IPIC UTC

University of Illinois Urbana-Champaign

Urbana, IL

TRANS-IPIC Quarterly Progress Report:

Project Description:

- Research Plan - Statement of Problem

Accurate and rapid installation of precast components is crucial to ensuring the quality and productivity of precast bridge construction. Typically, large prefabricated structural components are transported by cranes to the installation location for initial positioning. Then, multiple workers are needed to collaboratively adjust the component pose so that it can precisely align with the target. The workers will need to cooperate with the crane to place the component to its final position. In current practice, the performance of crane operation is affected by multiple factors, such as the size and weight of the lifting components, experience of crane operator, complexity of jobsite, and status of outdoor environments. As a result, repetitive re-lifting and falling may be needed, which is time-consuming and reduces construction efficiency. Furthermore, it requires close coordination between skilled operators and ground workers, which is labor-intensive when the entire industry is facing a significant labor shortage crisis. Robotic technologies have great potential to improve the accuracy and efficiency of precast component installation by automating crane operation. It was found that a partially automated tower crane, or even a simple observation device, could decrease the lifting time by 10% to 50% [1]. While existing research has developed methods for automated crane path planning, these approaches frequently overlook the inherent properties and kinematic constraints of the crane itself.

In our TRANS-IPIC Year 2 Exploratory project, we conducted a pilot study and developed a robotic AI agent using deep reinforcement learning (DRL) techniques and tested the feasibility of AI-based crane lift path planning for precast bridge component installation in a high-fidelity physics-based simulation environment. However, considering the heterogeneous site configurations and task specifications in bridge construction as well as the challenges and constraints on the jobsite, there is a critical need to generalize the method to various environmental and task configurations. Furthermore, the reliability and performance of AI-generated lift path planning should be evaluated both quantitatively and qualitatively by practitioners. The successful testing and demonstration of developed methods with domain experts will largely increase the feasibility of practical implementation, thus improving bridge construction efficiency and accuracy, saving costs, and benefiting long-term quality and durability of transportation infrastructure.

In existing studies, there are increasing applications of DRL in crane path planning [2–4]. However, there remain three knowledge gaps. First, the majority of research in automated crane path planning in AEC industry has historically concentrated on vertical construction projects within the building sector, leaving robotic installation in transportation infrastructure largely unexplored. Considering the substantial scale of precast components and the complex site configurations in horizontal projects, such as bridge construction, it is imperative to investigate the feasibility of robotic approaches to generate optimal crane lifting paths. Second, the design of appropriate reward functions for complex crane operations remains a significant challenge in DRL, as poor design can lead to non-executable lifting paths. Factors such as operator competency and situational awareness are difficult to capture through manually designed rewards [5]. Third, most existing methods are evaluated via computational simulation without understanding and evaluating in terms of feasibility and reliability in practice.

To this end, objective of this project is to develop and evaluate a practical-inspired and adaptive AI-based 3D crane lift path planning method for precast bridge installation. Building upon the foundational physics-based simulation and DRL agent developed in the exploratory project, this phase focuses on overcoming the gap between computational simulation and practical feasibility. The goal is to make technology adaptive to various job conditions, physical constraints, and practical industry needs.

- Research Plan - Summary of Project Activities (Tasks)

Task 1. Understand current practice and challenges in precast bridge installation

This task employs a mixed-methods approach of industry workshops and site observations to understand current practices. We will engage with industry professionals to brainstorm the usability of robotic installation and identify challenging jobsite environments. Furthermore, the research team will conduct site visits to observe crane operators, conduct semi-structured interviews, and collect site and task configuration data to understand key factors in lift path planning.

Task 2. Practice-oriented and adaptive AI-based lift path planning:

Building upon preliminary results, this task focuses on developing a DRL agent capable of generating optimal and safe lifting paths while adapting to various site and task configurations. The simulation environment will be expanded to include diverse site configurations and varying bridge geometries. Additionally, we will refine the reward function to incorporate factors like minimizing component swing and apply transfer learning techniques to ensure the model is generalized for different scenarios.

Task 3. Evaluation:

This task involves evaluating the developed method quantitatively and qualitatively by replicating a real-world case study project in our simulation. We will compare the performance of the AI-generated lift paths against those executed by human operators using metrics such as lifting time, path length, and placement accuracy. Qualitatively, we will engage with industry professionals to assess the perceived practicality and efficiency of the AI-based method.

Project Progress:

- Progress for each research task

Task1. Understand current practice and challenges in precast bridge installation [60% Completed]

- Industry Engagement & Data Collection:

  - We successfully initiated a collaboration with Zachry Construction, a major construction company specializing in highway and infrastructure construction, to understand the practical constraints and operational realities of precast bridge installation. We conducted an in-depth meeting with their team and acquired critical real-world project data.

- Site and Task Configuration:

  - From this collaboration, we acquired project materials for two distinct pre-cast bridge installation scenarios. We collected detailed 3D CAD models for a highway bridge project in San Antonio, which involves complex dual-crane (multi-agent) lifts slated for our future research phases (Figure 1). For our current single-agent RL framework, we acquired crane lifting sequence plans for another pre-cast bridge project in North Carolina. We also acquired specifications and lifting load charts for the specific crane models deployed on these sites. This data provides the vital physical parameters required to accurately scale our Isaac Sim environment.

Figure 1: A visualization of the 3D model from a pre-cast bridge project showing the site layout and precast components

- Girder Set Plans and Sequencing:

- Furthermore, we gathered Girder Set Plans and Girder Set Sequences detailing the strategic crane placement (e.g., designated Assembly Bases) and expected movement trajectories. This includes critical spatial constraints extracted directly from the plans, such as required operating radii and specific clearance zones, which directly inform the boundary conditions and penalty regions (negative rewards) in our RL agent's path planning trajectories.

Task 2: Practice-oriented and adaptive AI-based lift path planning [15% Completed]

- Simulation Environment Migration:

  - We transitioned our core physics-based simulation framework from MuJoCo to NVIDIA Isaac Sim (Figure 2). While the primary structural project files, 3D meshes, and kinematic environmental setups were successfully converted into the new platform, work is currently ongoing to remap and refine the material textures. Because Isaac Sim utilizes the Universal Scene Description (USD) format and an advanced physically-based RTX rendering pipeline, the imported visual assets and textures require manual adjustments to ensure a high-fidelity representation of the construction components.

Figure 2 Simulation Environment

- Advanced Computational Capabilities:

  - By leveraging Isaac Sim, we have enabled GPU-accelerated parallel training. This significantly increases the speed, scalability, and efficiency of the reinforcement learning process. The Proximal Policy Optimization (PPO) algorithms have been fully integrated and successfully executed within this new environment.

- Enhanced Physics Realism (Rope Simulation):

  - To bridge the "sim-to-real" gap and incorporate the real-world constraints identified by our industry collaboration, we initiated the development of an advanced rope simulation within the Isaac Sim physics engine. Technically, we are modeling the crane's wire utilizing sequential rigid-body segments (UsdGeom.Capsule) connected via UsdPhysics.SphericalJoint APIs. To ensure the heavy precast concrete component (e.g., modeled at 500 kg using MassAPI) exhibits realistic pendulum-like swaying rather than erratic spring-like bouncing, we implemented a custom DriveAPI on the joints. By explicitly configuring the damping attributes (set to 10.0) and nullifying the stiffness attributes (set to 0.0) across all rotational axes, the simulated components now react to gravity and crane movements.

Figure 3: A screenshot from Isaac Sim showing rope simulation

Task 3: Evaluation [0% Completed]

- Status:

  - This task has not yet commenced, as it relies on the maturation of the AI training currently underway in Task 2. Testing and validation will begin once the RL algorithms are adequately trained on the newly implemented physics.

- Percent of research project completed

By the first quarter, approximately 25% of the overall project is complete. This keeps the project on schedule for the first quarter and reflects our strong progress in Task 1 (60%), where we successfully initiated industry collaboration and acquired critical foundational data and operational constraints from our partner. It also accounts for the early-stage developmental progress in Task 2 (15%), marked by the successful migration to Isaac Sim and the commencement of advanced physics modeling. Task 3 remains at 0% as it is scheduled for the final phase of the project timeline.

- Expected progress for next quarter

Task 1: We are working with industry collaborator to identify ongoing pre-cast bridge projects around San Antonio for site visits. In the next quarter, we plan to observe the crane operation during bridge installation and meet with site personnel, such as crane operators, safety supervisors, spotters (depending on their availability) to understand the operational consideration during bridge installation. The findings will inform the design of AI model for crane path planning.

Task 2: Our primary objective for the upcoming quarter is to complete and validate the multi-link rope simulation. We will expand the current single-link prototype into a fully articulated multi-link chain to accurately reflect variable cable lengths and environmental disturbances like wind forces. Once this complex physics environment is finalized, we will implement transfer learning by loading pre-trained weights from the previous phase. Additionally, we will refine the reward function to specifically penalize the excessive sway and jerky movements generated by the new rope dynamics, thereby improving the adaptive agent's real-world robustness.

Task 3: While formal evaluation is planned for later in the year, we will begin gathering and preparing the ground-truth data, e.g., human operator logs during bridge installation, to ensure it is ready for metric comparison (Lifting Time, Path Length, Placement Accuracy) once the RL agent matures.

- Educational outreach and workforce development

No activities occurred this quarter. Future efforts may include student training or workshops tied to project advancements.

- Technology Transfer

No technology transfer activities took place this quarter. Future plans may involve sharing the simulation environment and/or RL model with industry stakeholders for feedback and improvement.

Research Contribution:

- Papers that include TRANS-IPIC UTC in the acknowledgments section:

N/A

- Presentations and Posters of TRANS-IPIC funded research:

We have just presented the exploratory project from Year 2 (this current project is a continuation of it) in a major international conference in construction engineering and management. Details are as follows:

- Date: March 20, 2026

- Conference: CI & CRC Joint Conference 2026

- Location: San Antonio, TX

- Presentation:

  - Mohsen Navazani, Ao Du, Shuai Li, and Jiannan Cai. “Reinforcement Learning-Based Motion Planning for Mobile Crane Operation in Precast Concrete Bridge Installation”

- The corresponding paper is accepted for proceedings and is in press.

- Please list any other events or activities that highlights the work of TRANS-IPIC occurring at your university (please include any pictures or figures you may have). Similarly, please list any references to TRANS-IPIC in the news or interviews from your research.

N/A

Appendix 1: Research Activities, leadership, and awards (cumulative, since the start of the project)

- Number of presentations at academic and industry conferences and workshops of UTC findings

- No. = 1

- Number of peer-reviewed publications submitted based on outcomes of UTC funded projects

- No. = 0

- Number of peer-reviewed journal articles published by faculty.

- No. = 0

- Number of peer-reviewed conference papers published by faculty.

- No. = 0

- Number of TRANS-IPIC sponsored thesis or dissertations at the MS and PhD levels.

- No. MS thesis = 0

- No.  PhD dissertations = 0

- No. citations of each of the above = 0

- Number of research tools (lab equipment, models, software, test processes, etc.) developed as part of TRANS-IPIC sponsored research

- Research Tool #1 (Name, description, and link to tool) = 1 (Isaac simulation environment, still under development)

- Research Tool #2 (Name, description, and link to tool) = 0

- Research Tool #3 (Name, description, and link to tool) = 0

- Number of transportation-related professional and service organization committees that TRANS-IPIC faculty researchers participate in or lead.

- Professional societies

  - No. participated in = 0

  - No. lead = 0

- Advisory committees (No. participated in & No. led)

  - No. participated in = 0

  - No. lead = 0

- Conference Organizing Committees (No. participated in & No. led)

  - No. participated in = 0

  - No. lead = 1

- Editorial board of journals (No. participated in & No. led)

  - No. participated in = 0

  - No. lead = 0

- TRB committees (No. participated in & No. led)

  - No. participated in = 0

  - No. lead = 0

- Number of relevant awards received during the grant year

- No. awards received = 0

- Number of transportation related classes developed or modified as a result of TRANS-IPIC funding.

- No. Undergraduate = 0

- No. Graduate =  0

- Number of internships and full-time positions secured in the industry and government during the grant year.

- No. of internships = 0

- No. of full-time positions = 0

References:

[1]	S.H. Cho, S.U. Han, Reinforcement learning-based simulation and automation for tower crane 3D lift planning, Automation in Construction 144 (2022). https://doi.org/10.1016/j.autcon.2022.104620.

[2]	O. Makarov, T. Harada, Reinforcement Learning for AI-Positioning of Suspended Load by a Crane Vessel, in: 2024 10th International Conference on Electrical Engineering, Control and Robotics, EECR 2024, 2024. https://doi.org/10.1109/EECR60807.2024.10607270.

[3]	D.H. Chun, M. Il Roh, H.W. Lee, Automation of crane control for block lifting based on deep reinforcement learning, J. Comput. Des. Eng. 9 (2022). https://doi.org/10.1093/jcde/qwac063.

[4]	A. Zhu, T. Dai, G. Xu, P. Pauwels, B. De Vries, M. Fang, Deep Reinforcement Learning for Real-Time Assembly Planning in Robot-Based Prefabricated Construction, IEEE Transactions on Automation Science and Engineering 20 (2023). https://doi.org/10.1109/TASE.2023.3236805.

[5]	Z. Wang, C. Huang, B. Yao, X. Li, Integrated reinforcement and imitation learning for tower crane lift path planning, Automation in Construction 165 (2024). https://doi.org/10.1016/j.autcon.2024.105568.